|  | Yes | No | N/A |
| --- | --- | --- | --- |
| - Is your local fire department well acquainted with your facilities, its location and specific hazards? |  |  |  |
| - If you have a fire alarm system, is it certified as required? |  |  |  |
| - If you have a fire alarm system, is it tested at least annually? |  |  |  |
| - If you have interior standpipes and valves, are they inspected regularly? |  |  |  |
| - If you have outside private fire hydrants, are they flushed at least once a year and on a routine preventive maintenance schedule? |  |  |  |
| - Are fire doors and shutters in good operating condition? |  |  |  |
| - Are fire doors and shutters unobstructed and protected against obstructions, including their counterweights? |  |  |  |
| - Are fire door and shutter fusible links in place? |  |  |  |
| - Are automatic sprinkler system water control valves, air and water pressure checked weekly/periodically as required? |  |  |  |
| - Is the maintenance of automatic sprinkler systems assigned to responsible persons or to a sprinkler contractor? |  |  |  |
| - Are sprinkler heads protected by metal guards, when exposed to physical damage? |  |  |  |
| - Is proper clearance maintained below sprinkler heads? |  |  |  |
| - Are portable fire extinguishers provided in adequate number and type? |  |  |  |
| - Are fire extinguishers mounted in readily accessible locations? |  |  |  |
| - Are fire extinguishers recharged regularly and noted on the inspection log? |  |  |  |
| - Are employees periodically instructed in the use of extinguishers and fire protection procedures? |  |  |  |

Fire Extinguishers:

Discharged hose/nozzle is in good shape and not clogged, cracked, or broken.

It is mounted in an easily accessible area, with nothing stacked around it.

Safety Pin is in place and not damaged.

Pressure gauge is in the green and not damaged or showing “recharge”.

Label is readable and displays the type of extinguisher and the instructions for use.

It is not rusty, or has any type of corrosion build up.

Extinguisher was turned upside down at least three times. (Shaken)

The location of the extinguisher is easily identifiable. (Signs)

| Audited by (print) |  | Date |  | Signed |
| --- | --- | --- | --- | --- |